Using the Digital

Microscope

Getting Started

- You will be using the program ISCapture.  If it is not already running, click on the orange ISCapture icon on your laptop screen.

- Turn on the green switch on the back of the microscope.

- Make sure your microscope is on the red 4X objective (if not tell a teacher).

- Place your slide in the mechanical stage and center the specimen.

- Use the coarse adjustment to move the stage up until it meets the stop.

- Use the coarse adjustment to turn the stage down until the image comes in focus.

- Use the fine adjustment to reach the best focus.

- On the computer, press the “White Balance” button to automatically adjust color.

Using the computer

- If the image looks good press the “Capture” button.  If not use the steps below. Remember if you want to change power to get a better picture, ask a teacher.

Adjusting Exposure

- Turn off Auto Exposure on the left of your computer screen.

- Use the following to adjust the light – 1. exposure slider on computer 2.  wheel on base of the microscope which controls the lamp  3. the diaphragm

- Turn on the Auto Exposure when moving to a new power or finishing for the day.

Making your report

- After you have two good pictures, minimize the program.

- Open the desktop file called “Microscope Report” and shrink the window.

- Your pictures are in a folder called “ISCapture Images.” Open it and shrink the window.

- Drag your 2 best pictures over to the “Microscope Report” page

- Label the organisms and add the names of the people in your lab group

- Print the page (it may take several minutes to print)

- When the page prints, close the file but when it asks if you want to save say “no.”

- Look at other organisms until the end of the period

Preparing for the next class

- Open the “ISCapture Images” folder that contains your pictures

- Make sure the “Home” tab is selected at the top of the page

- Click on “select all” at the right top of the page

- Click on the red X delete button

- All your pictures should now be deleted

- Open the ISCapture so the next class will be ready to go

- Make sure the microscope is on the 10X objective