Procedure for Identification of Adhesive Tape Lab:

## Collect a background spectrum (only if you are the first group of the day)

- Remove any sample from the sample compartment or accessory.

- Using the mouse, click on Collect, Background.

- Enter a spectrum title.  Suggested title:  initials bg date

- Choose OK.  A gauge shows the progress of collection.

Collect a sample spectrum

- Place sample of tape on sample compartment or accessory adhesive side down.

  - If using a swap-top accessory, turn and pull the silver release knob at the back of the swap-top (on the right side of the accessory).

  - Once the release mechanism is disengaged, tilt the pressure device out of the way.  Tilt it back until the silver release knob clicks into the back position

  - Place sample on the base of the accessory so that it lies on the small hole in the base.

  - While holding the pressure device, pull the silver knob out and move it to the upright position so the tip is now over the sample.

  - Turn the top knob clockwise until it clicks.  The instrument automatically determines the appropriate pressure necessary to run the sample.

- Press Collect, Sample

- Enter a spectrum title.  Suggested title: Adhesive type, initials, date

- Before it is displayed, the sample spectrum is automatically ratioed against the current background.  In other words, the background is subtracted from the sample.

- Print your spectra by selecting the File button, choose Print, choose Landscape, and then hit OK.

Note:  The subtraction command is only used to subtract one sample from another.  Unlike other infrared specs, the background scan is automatically ‘ratioed’ or ‘ or ‘subracted’ from the sample scan.